Smart Display  ·  Group 5א  ·  IoT project 236333, Technion

Edge Cases Design

Week 10 submission: un-planned events the display and the app are likely to meet, and how each one is handled

Shada Nujeidat, Yuval Carmelli, Liat Rokach  ·  11 October 2026

An edge case is an event we did not plan as the normal flow but that is likely to happen: WiFi drops, a service is down, the user taps the wrong thing, a command arrives at an awkward moment, the power goes out. For each one we define a user indication (how the user knows there is a problem) and a recovery behavior (how the system keeps working). The last column shows where it is handled in the code (firmware files are under main/src/; app files are in the phone app repository).

| Handled | implemented and tested on the board |
| --- | --- |
| Partly | works, but the indication or the recovery can be better; the planned change is in the last column |
| Planned | not handled yet; fix planned before the final submission |
| Accepted | left as is on purpose (rare, low impact), documented |

# 1. Edge cases and responses

## A. Network and WiFi

| # | Edge case | User indication | Recovery behavior | Status | Where |
| --- | --- | --- | --- | --- | --- |
| 1 | First start, no WiFi set up | Header shows "⚠ Offline". Clock: "Waiting for WiFi". Weather card: "No WiFi" | The display starts right away and works offline. A setup portal (WiFi network SmartDisplay-Setup) runs in the background; the user joins it from a phone and picks a network. No re-flashing needed | Handled | wifi_setup.h |
| 2 | WiFi drops while running | Header turns to "⚠ Offline" within 2 s on every screen. Weather "Updated 10:30" turns red (data is old) | Last weather, events and lists stay on screen. The saved network is retried every 15 s. Everything that does not need internet keeps working | Handled | connection_status.h, wifi_setup.h |
| 3 | Saved network gone for good (moved to another place, router changed) | "⚠ Offline" | After 1 min offline the setup portal opens again, so a new network can be chosen. While it is open the saved network is retried every 60 s instead of 15 s so the portal stays responsive. It closes by itself when connected | Handled | wifi_setup.h |
| 4 | Wrong WiFi password typed in the portal | Header stays "⚠ Offline" | The setup network stays available, so the user joins it again and re-enters the password | Handled | wifi_setup.h |
| 5 | WiFi connected but no internet (captive portal, e.g. a login page) | WiFi icon shows connected. Weather keeps its old "Updated" time; Calendar: "Couldn't load events / Trying again every minute" if it has no events yet | Weather: one immediate retry, then the regular schedule (every 30 s until the first success, then every 10 min). Calendar retries every 60 s. Firebase retries every second and logs the error once | Partly | weather.h, calendar.h Next: show "No internet" in the header when every service fails |
| 6 | WiFi drops in the middle of a request | Screen may pause for a few seconds | TLS handshake limit 8 s (15 s for Firebase) instead of the default 120 s. Firebase runs in its own task on core 0 and never blocks the screen | Partly | weather.h, calendar.h Next: move weather and calendar downloads to the core-0 task |
| 7 | Network blocks time servers (Technion) | Clock shows "--:--" and "Getting the time..." | ntp.technion.ac.il is asked first, pool.ntp.org and time.google.com are backups | Handled | time_date.h |

## B. Online services (weather, calendar, Firebase)

| # | Edge case | User indication | Recovery behavior | Status | Where |
| --- | --- | --- | --- | --- | --- |
| 8 | Weather service down, error reply, or broken JSON | Before the first success: "Loading..." on the card and screen. After: the old data with its "Updated" time | Nothing is overwritten by a bad reply. Retried as above. One request brings all 5 cities, so switching city never needs the network | Handled | weather.h |
| 9 | Calendar not configured (no ID / API key) | Calendar: "Calendar not set up" with a hint to add them; home card: "Not set up" | Everything else works normally | Handled | calendar.h, calendar_screen.h |
| 10 | Wrong calendar ID or API key | "Couldn't load events" with a specific hint: "Check CALENDAR_ID, and that the calendar is public" (404) or "Check CALENDAR_API_KEY" (400/403) | Retried every minute; events from the last successful check stay on screen | Handled | calendar_screen.h |
| 11 | No upcoming events | "No upcoming events / Your calendar is clear" | - | Handled | calendar_screen.h |
| 12 | Event title in Hebrew, very long, or with emoji | Hebrew shown right-to-left; long titles cut with "…" on one line; characters the fonts lack shown as "?" (never as empty boxes) | - | Handled | shared_ui.h |
| 13 | Firebase sign-in fails (wrong secrets, sign-in method off) | On the display: none, only in the Serial log. In the app: "Not confirmed" after 60 s | Sign-in retried every 30 s; the rest of the display works | Partly | firebase_sync.h Next: small "voice commands unavailable" icon in the header |
| 14 | Firebase login expires (tokens last 1 hour) | None | Renewed 5 minutes before it expires; if a request still gets "expired", it signs in again on the next round | Handled | firebase_sync.h |
| 15 | Firebase unreachable | App: "Not confirmed" | Retried every second, error logged once. List changes made meanwhile are kept and copied to /lists when the connection is back | Handled | firebase_sync.h |

## C. Voice commands (phone app → Firebase → display)

| # | Edge case | User indication | Recovery behavior | Status | Where |
| --- | --- | --- | --- | --- | --- |
| 16 | Phone is offline | App: "You're offline. Connect to the internet to send voice commands." Recording is refused | Banner disappears when the phone is back online | Handled | app: pwa.js, app.js |
| 17 | Microphone permission denied, or app opened without HTTPS | "Could not open your microphone. Check browser permissions." / "Open this app over HTTPS in Safari or Chrome…" | Nothing is sent; the user fixes the permission and tries again | Handled | app.js |
| 18 | Very long recording | Recording stops | Stops automatically after 30 s and is sent | Handled | app.js |
| 19 | Gemini does not understand, or it is not a list command | "Could not understand a valid command. Please try again." | The server checks the JSON (action, list, non-empty item) and answers 422; nothing is written to Firebase, the display is not touched | Handled | server.js, commands.js |
| 20 | Gemini busy (503) | "Understanding your request…" takes longer | The server retries 3 times, 2 s apart, then reports an error | Handled | server.js |
| 21 | Display off or offline when a command is sent | App waits 60 s, then "Not confirmed - Check your list before trying again" | When the display starts, it skips the command already waiting, so an old command is never applied long after the user gave up | Handled | commands.js, firebase_sync.h |
| 22 | Item already on the list ("add milk" twice) | Pop-up "milk is already on your shopping list"; app shows success | No duplicate. Matching ignores upper/lower case | Handled | lists.h |
| 23 | Remove an item that is not on the list | Red pop-up "eggs isn't on your shopping list"; app shows failure | List unchanged | Handled | lists.h, firebase_sync.h |
| 24 | List full (20 items) | Red pop-up "Your shopping list is full" | Item not added; the user ticks items off to make room | Handled | lists.h |
| 25 | Item name too long | Shown on one line with "…" | Stored up to 47 bytes, cut at a letter boundary so Hebrew letters are never broken | Handled | lists.h |
| 26 | Empty item, unknown list or action (bad JSON written to /command) | Red pop-up "The display couldn't understand this command"; app shows failure | Lists unchanged | Handled | firebase_sync.h |
| 27 | Several commands in a row | Each opens its list with its pop-up | Every command is applied to the list immediately; screen updates wait in a queue (4 places). If the screen falls behind, extra pop-ups are dropped but the lists are still correct | Handled | firebase_sync.h |
| 28 | Two phones send at the same moment | Each app waits for the reply with its own number | The command number (runningCount) is written in a transaction; a second command can still overwrite the first one in the single /command slot | Accepted | server.js |
| 29 | /command deleted in the console | None | Numbering restarts at 1; the display compares "different from the last one", not "greater than", so it keeps working | Handled | firebase_sync.h |
| 30 | Command arrives while the user is using another screen (e.g. Settings) | The list it changed opens with the pop-up | Leaving a screen first saves its state. If nobody touches the list, it goes back home after 30 s | Partly | display_ui.h Next: if the screen was touched in the last few seconds, show only the pop-up |
| 31 | Voice "add milk" while "milk" is ticked (2 s undo window) | Pop-up "Added milk…" / "already on" | The pending tick removes by name, so the wrong item is never removed, but milk may disappear after the 2 s | Planned | lists_screen.h Next: cancel a pending tick when a voice command names the same item |

## D. Touch screen and user interface

| # | Edge case | User indication | Recovery behavior | Status | Where |
| --- | --- | --- | --- | --- | --- |
| 32 | Touch while the screen is dimmed or off | Screen wakes to the saved brightness | The waking touch does not press anything under the finger | Handled | idle.h |
| 33 | Tap registered twice, or a scroll taken as a tap (resistive touch) | None | LVGL acts on one complete press-and-release and ignores a press that turns into a scroll; buttons are at least 44 px | Handled | display_ui.h |
| 34 | Item ticked by mistake | Row crossed out | Removed only after 2 s; tapping it again keeps it | Handled | lists_screen.h |
| 35 | Brightness slider pulled to the bottom | Screen stays dimly lit | Minimum is 10/255 (4 %), so the screen can never be turned fully dark by mistake | Handled | settings.h |
| 36 | Empty list | "Shopping list is empty" / "Nothing to do" | - | Handled | lists_screen.h |
| 37 | Text longer than its space (city, item, event) | One line with "…"; Hebrew right-to-left | - | Handled | shared_ui.h |
| 38 | Board not touched for a long time | Dims or turns off after the chosen time (1-30 min) if power saving is on | Any touch wakes it | Handled | idle.h |

## E. Data, storage and power

| # | Edge case | User indication | Recovery behavior | Status | Where |
| --- | --- | --- | --- | --- | --- |
| 39 | Power cut or reset | Normal start; clock "Getting the time..." until synced | Settings and both lists are saved in flash and restored. Time is synced again from NTP (the board has no clock battery) | Handled | storage.h, lists.h |
| 40 | List changed without WiFi (instructor feedback) | None needed: the change shows at once | Saved in flash and marked with a new version number; copied to Firebase /lists as soon as WiFi is back | Handled | lists.h, firebase_sync.h |
| 41 | Saved data from an older firmware, or out-of-range saved settings | Lists start empty (old format) or the setting shows its default | Saved lists are checked by size and format version and ignored if they don't match; every saved setting is limited to its valid range | Handled | lists.h, settings.h, idle.h |
| 42 | Flash write fails | Only in the log: "[Lists] Could not save to flash" | The list stays correct in memory; saved again at the next change | Partly | lists.h |
| 43 | Clock not synced yet | "--:--" and "Waiting for WiFi" / "Getting the time..."; world clock without hands | Calendar waits for the time before loading, so "Today" is never wrong | Handled | time_date.h, calendar.h |
| 44 | Summer / winter time change | None | Israel's rule and each world-clock city's own rule are built in; the change happens automatically | Handled | time_date.h |
| 45 | Memory runs low or a task runs out of stack | Board restarts | Fixed-size buffers for lists and events; weather and calendar never run at the same time; free memory printed at each sign-in; after a restart all state comes back from flash | Handled | firebase_sync.h |
| 46 | Weak USB power (laptop port, long cable) | Board restarts when WiFi and backlight peak | All state is restored after the restart. Use a 5 V / 1 A adapter for the display | Accepted | README |

# 2. Robustness self-check (6 criteria from the course guide)

| Criterion | What the project has | Gap / next step |
| --- | --- | --- |
| 1. Feature-rich | LVGL touch UI with 6 screens, voice commands through Gemini and Firebase, Google Calendar, weather for 5 cities in one request, world clock with analog dials, Hebrew right-to-left text, power saving. Firebase runs in its own FreeRTOS task on the second core | Move weather/calendar downloads to the background task (performance) |
| 2. Generic / modular | One folder per feature with the same pattern (init<Feature>(), handle<Feature>(), build<Name>Screen()); data modules contain no drawing code; all colors and fonts come from one design system (theme.h); hard-coded values listed in Documentation/parameters.md | Optional "demo data" mode for showing the UI without internet |
| 3. Configurable | WiFi through the setup portal (no re-flashing); city, 24-hour time, brightness, dark/light mode and power saving from Settings, all saved; secrets in secrets.h with a template | Firebase login and calendar ID still need a re-flash (could move to the setup portal). Touch calibration is fixed; test_touch prints the raw values to re-calibrate |
| 4. Stable | Non-blocking WiFi, timeouts on every request, retries, offline mode with data kept in flash, lists protected by a lock, LVGL used from one task only, unit tests for every part | Technician / debug page in Settings (IP, signal, free memory, last errors) instead of only the Serial log |
| 5. Organized | Code split by feature with comments; only handleTouch() changes the open screen; each hardware part is accessed by one module; single board, no external wiring | Enclosure (see Enclosure Sketch) |
| 6. User-friendly | Every state is shown on screen (Offline, Loading, Not set up, Updated time, red/blue pop-ups); the app shows the result of each command; wrong input is rejected with a clear message; scrollable lists and a status bar on every screen | Tested with family and friends; changes made from their feedback (see Feasibility Tests 3.4). Next: another short test of the final version |

Totals: 46 edge cases. Detailed messages and log lines for each state: Documentation/errors_and_states.md in the firmware repository.